# Pan-Tilt-Laser Wiring & Setup Guide

# Hardware

- Arduino Nano (new bootloader / CH340 USB chip)

- 2x Continuous rotation servo motors (pan and tilt)

- 1x Laser module (active LOW)

# Servo Motor Wiring

Each servo has 3 wires:

| Wire Color | Connect To |
| --- | --- |
| Red (VCC) | Nano 5V pin |
| Black/Brown (GND) | Nano GND pin |
| Yellow/Orange (Signal) | See signal pin table below |

| Servo | Signal Pin |
| --- | --- |
| Pan | D9 |
| Tilt | D10 |

IMPORTANT: Red and black wires MUST connect to the dedicated 5V and GND pins on the Nano, NOT into numbered digital pins. Digital pins cannot supply enough current (servo needs ~500mA, pins only give ~40mA).

# Laser Module Wiring

| Wire | Connect To |
| --- | --- |
| VCC | Nano 5V |
| GND | Nano GND |
| Signal | D5 |

IMPORTANT: This laser module is active LOW — it turns ON when pin is LOW and OFF when HIGH. The firmware handles this correctly.

# PlatformIO Configuration

Board: nanoatmega328new (new bootloader)

Platform: atmelavr

Framework: arduino

Library: arduino-libraries/Servo@^1.2.1

Monitor baud rate: 9600

Upload port: COM5 (auto-detected)

# Serial Monitor Controls

| Key | Action |
| --- | --- |
| w | Tilt continuous up |
| s | Tilt continuous down |
| a | Pan continuous left |
| d | Pan continuous right |
| i | Nudge tilt up |
| k | Nudge tilt down |
| j | Nudge pan left |
| l | Nudge pan right |
| o | Laser ON |
| f | Laser OFF |

Space or X: Stop all motors

# Calibration Values (in main.cpp)

panStop = 90        (servo stop position for continuous rotation)

tiltStop = 90       (servo stop position for continuous rotation)

speedOffset = 15    (higher = faster continuous movement)

nudgeTime = 55      (milliseconds per nudge)

# Upload Commands

Upload firmware:

pio run --target upload

Open serial monitor:

pio device monitor --port COM5 --baud 9600