Raspberry Pi 5 Setup Documentation: Bird Vision Node

Overview This document outlines the steps taken to prepare and configure the Raspberry Pi 5 for the bird detection vision system. The setup was performed completely "headless" (without a monitor or keyboard attached to the Pi) to streamline the deployment process.

Prerequisites

- Raspberry Pi 5

- MicroSD Card and adapter

- Laptop (Host machine)

- Raspberry Pi Imager software

- Visual Studio Code (VS Code) with the "Remote - SSH" extension installed

Step 1: Flashing the Operating System

To ensure compatibility with our specific camera libraries and hardware, the OS was flashed manually using the host laptop.

- Inserted the microSD card into the laptop.

- Opened the Raspberry Pi Imager tool.

- For the device, selected Raspberry Pi 5.

- For the Operating System, selected Raspberry Pi OS (Legacy, 64-bit).

- Before flashing, clicked the gear icon (OS Customization settings) to pre-configure the setup:

  - Enabled SSH using password authentication.

  - Set the username to birdguard_1.0 and assigned a secure password.

  - Configured the Wireless LAN with the local Wi-Fi network name (SSID) and password.

  - Set the local time zone settings.

- Clicked "Write" to burn the OS to the microSD card. Once completed, the card was safely ejected.

Step 2: Booting and Network Discovery

With the OS fully pre-configured, the physical setup of the Pi was minimal.

- Inserted the flashed microSD card into the Raspberry Pi 5.

- Connected the power supply and turned the Pi on.

- Because the Wi-Fi credentials were saved during Step 1, the Pi automatically connected to the local network upon booting.

- Logged into the local Wi-Fi router's admin dashboard to view connected devices and located the IP address assigned to the Raspberry Pi.

Step 3: Initial Remote Access

To verify the system was online and accessible, an initial connection was made from the host laptop's terminal.

- Opened the terminal on the laptop.

- Connected to the Pi using the assigned IP address and the configured username:

Bash

ssh birdguard_1.0@<insert_pi_ip_address>

- Accepted the security fingerprint and entered the password to successfully access the Pi's command line.

Step 4: Development Environment Setup

To finalize the setup and get the code running, VS Code was used to bridge the host laptop and the Pi.

- Opened VS Code on the laptop.

- Used the Remote - SSH extension to connect to birdguard_1.0@<insert_pi_ip_address>. This allowed direct editing of the Pi's file system through the laptop's VS Code interface.

- Opened a terminal within VS Code (which is now running directly on the Pi) and cloned the project repository:

Bash

git clone <insert_repository_url>

- Navigated into the project directory and used uv (the ultra-fast Python package installer) to create a virtual environment and install all dependencies required by the DepthAI and vision scripts:

Bash

cd <project_folder_name>

uv venv

source .venv/bin/activate

uv pip install -r requirements.txt

(Note: Any additional system level libraries like OpenCV dependencies were installed via sudo apt install as required by the vision setup).

The Raspberry Pi 5 is now fully configured, connected to the network, and ready to execute the vision detection scripts using the uv run command.